| Name | Ashwin Varkey | Chukwudi Williams | Manik Malik |
| --- | --- | --- | --- |
| Matriculation No. | 23115420 | 23061170 | 23169717 |

Assignment 7 – Basic Open MP

1. Machine Topology

(a)No of cores in one node = 72

(b)RAM Size = 503.5 GB (Front end node)

RAM Size = 251.580 GB (Compute node)

(c)L1 cache- each core has its own

Size 48 KB*72=3.456 MB

L2 Cache-each core has its own

Size 1.25 MB*72=90 MB

L3 Cache

Size 54 MB

All the cores share L3 cache (Shared for each socket)

L3 is shared by 36 cores (each socket)

(d)Fritz has 2 sockets and 4 ccNUMA domains

2a) Parallel Update Benchmark Fixed Frequency@2 GHz

The performance does increase proportionally with increase in number of cores till L3 cache memory capacity. It is inferenced that under higher number of iterations in the range of 10^5 to 10^7 there is a significant amount of speedup when multithreading is implemented

2 b)

To analyze the scalability of the benchmark for different working set sizes, we need to compare the performance of the parallel code with the purely serial code.

From the results obtained in part (a), we can compare the performance of the serial code (one thread) with the parallel code (multiple threads) for different loop lengths.

If the speedup approaches the number of threads used, it indicates good scalability, suggesting that n cores can indeed compute n times faster.

But let’s take the case of thread 16 in which case we get a speed up of around 6.5 (not 16) which suggests that the memory hierarchy, including L1 cache, L2 cache, L3 cache, and main memory, also plays a crucial role in determining the scalability.

For thread sizes 8 and 16 we get good speed up with multiple threads

Smaller working sets that can fit entirely within the L1 or L2 cache have a higher likelihood of exhibiting better scalability with multiple threads. This is because the cache latency is significantly lower compared to accessing data from main memory. When the working set size exceeds the cache capacity, frequent cache evictions and main memory accesses can introduce more contention and limit scalability.

However, if the speedup is significantly lower than the number of threads, it suggests that the benchmark is constrained by factors such as cache sizes or memory bandwidth, and n cores may not achieve a linear increase in performance.

The effectiveness of parallelization also depends on the nature of the benchmark and array length itself.

Code:

#include <iostream>

#include “timing.h”

#include <stdio.h>

#include <stdlib.h>

#include <omp.h>

int main()

{

double wct_start, wct_end;

int id, nt, k, i;

int NITER, N;

double t = 0;

const double s = 1.00000000001;

const int maxThreads = 16;

const int numMeasurements = 10;

int totalGflops;

// Loop length N range: 10 to 100,000,000

for (int N = 10; N <= 100000000; N *= 1.2)

{

std::cout << "Loop length (N): " << N << std::endl;

// Perform measurements for different thread counts

for (int numThreads = 1; numThreads <= maxThreads; numThreads *= 2)

{

double totalGflops = 0.0;

NITER = 1;

// Run the benchmark multiple times for accurate results

// Initialize arrays a[] and b[]

double a[N], b[N];

for (int i = 0; i < N; ++i)

{

a[i] = 0.; // Initial values of array a[]

b[i] = 0.; // Initial values of array b[]

}

// Measure execution time using chrono library

NITER = 1;

do {

// Time measurement

wct_start = getTimeStamp();

// Repeat measurement often enough

for (k = 0; k < NITER; ++k) {

// This is the benchmark loop

#pragma omp parallel for num_threads(numThreads)

for (i = 0; i < N; ++i) {

// Update benchmark

a[i] = s * a[i] + b[i];

}

// End of benchmark loop

if (a[N / 2] < 0.)

printf("%lf", a[N / 2]);

}

wct_end = getTimeStamp();

NITER = NITER * 2;

} while (wct_end - wct_start < 0.2);  // At least 100 ms

NITER = NITER / 2;

double time = (wct_end - wct_start) / NITER;

double MFlop = (2.0 * N * NITER) / (time * 1.0e9);

double MFlopPerSec = MFlop / (1.0e-3 * NITER);

// Output the results

std::cout << "Threads: " << numThreads << ", Gflop/s: " << MFlopPerSec << std::endl;

}

}

return 0;

}

2 c)

To calculate the maximum memory bandwidth achieved, we need the performance data in Gflop/s for 1 to 18 cores and then taking maximum performance

Maximum Memory Bandwidth = Max Performance * Data transferred per Flop

4.48*16=71.68 GB/s

Code:

#include <stdio.h>

#include <stdlib.h>

#include <time.h>

#include <omp.h>

double getTimeStamp()

{

struct timespec ts;

clock_gettime(CLOCK_MONOTONIC, &ts);

return (double)ts.tv_sec + (double)ts.tv_nsec * 1.e-9;

}

int main()

{

const int N = 100000000;

double wct_start, wct_end;

const double s = 2.0;

const int cores = 18;

const int numMeasurements = 10;

// Perform measurements for different cores

for (int numThreads = 1; numThreads <= cores; numThreads ++)

{

double totalGflops = 0.0;

// Run the benchmark multiple times for accurate results

for (int measurement = 0; measurement < numMeasurements; ++measurement)

{

// Initialize arrays a[] and b[]

double a[N], b[N];

for (int i = 0; i < N; ++i)

{

a[i] = 0.; // Initial values of array a[]

b[i] = 0.; // Initial values of array b[]

}

wct_start = getTimeStamp();

// Parallelize the update loop with OpenMP

#pragma omp parallel for num_threads(numThreads)

for (int i = 0; i < N; ++i)

a[i] = s * a[i] + b[i];

wct_end = getTimeStamp();

double time = (wct_end - wct_start);

// Calculate Gflop/s

double gflops = (2*N) / (time * 1e6);

totalGflops += gflops;

}

// Average Gflop/s across measurements

double avgGflops = totalGflops / numMeasurements;

// Output the results

printf("%d\t %f\n", numThreads, avgGflops/1e3);

}

return 0;

}